Executing a complete instruction in a computer's Central Processing Unit (CPU) involves a cycle of three fundamental steps: Fetch, Decode, and Execute. This is often referred to as the Instruction Cycle or Fetch-Decode-Execute Cycle.

Here is an illustration of the execution of a simple instruction: ADD R1, R2 (Add the contents of Register R2 to Register R1 and store the result in R1).

## 1 The Instruction Cycle

| Stage | Description | Key Components Involved |
| --- | --- | --- |
| Fetch | The CPU retrieves the next instruction from the memory location pointed to by the Program Counter (PC) and loads it into the Instruction Register (IR). The PC is then incremented. | PC, Memory Address Register (MAR), Memory Data Register (MDR), IR. |
| Decode | The Control Unit (CU) interprets the instruction in the IR to determine the operation to be performed and the addresses of the operands. | IR, Control Unit (CU). |
| Execute | The Arithmetic Logic Unit (ALU) performs the operation specified by the instruction using the decoded operands, and the result is stored back in the destination (memory or register). | ALU, Registers. |

## 2 Example: Executing ADD R1, R2

Assume the following initial state:

- Program Counter (PC): Points to memory address 0x100 (where the instruction is stored).

- Memory at 0x100: Contains the instruction ADD R1, R2.

- Register R1: Contains the value 5.

- Register R2: Contains the value 3.

### Phase 1: FETCH

- The CPU copies the address from the PC (0x100) into the Memory Address Register (MAR).

- The CPU issues a read command to memory.

- The instruction ADD R1, R2 is read from memory and loaded into the Memory Data Register (MDR).

- The instruction is transferred from the MDR to the Instruction Register (IR).

- The PC is incremented to point to the next instruction (0x104, assuming 4-byte instructions).

### Phase 2: DECODE

- The Control Unit (CU) reads the instruction ADD R1, R2 from the IR.

- The CU determines that the operation is addition (ADD).

- The CU identifies the operands: the contents of R1 and R2.

- The CU sets up the necessary control signals for the execution phase (e.g., instructing the ALU to perform addition and selecting R1 and R2 as inputs).

### Phase 3: EXECUTE

- The contents of R1 (5) and R2 (3) are sent as inputs to the Arithmetic Logic Unit (ALU).

- The ALU performs the addition: 5+3=8.

- The result (8) is written back to the destination register, which is R1.

### Final State

- Program Counter (PC): 0x104 (ready for the next instruction).

- Register R1: Now contains the new value 8.

- Register R2: Still contains the value 3.

The cycle is now complete, and the CPU is ready to start the Fetch stage for the instruction at 0x104.